CUMCM-2016 Problem C

Prediction of Battery Remaining Discharge Time

As a power source, lead-acid batteries are widely used in industry, military, and daily life. In the process of constant current discharge of lead-acid battery, voltage decreases with discharge time until the rated minimum protection voltage (Um, 9V in this problem). From the beginning of the full charge state, the relationship between the voltage and the time is called the discharge curve. One may ask a question: How long the battery can supply under the current load (i.e., the discharge time of the battery discharging to Um with the current constant current intensity). In addition, the charge state of the battery will attenuate after long time use or storage.

Problem1  Appendix 1 is the sampling data of the complete discharge curves of the same production batch battery with different current intensity. Please use the elementary function to represent each discharge curve according to Appendix 1, and give the mean relative error (MRE, defined in Appendix 1) of each discharge curve. If new batteries discharge with current intensity of 30A, 40A, 50A, 60A, or 70A, respectively, and in each case the measured voltage is 9.8V, please give the remaining discharge time of the batteries according to your model.

Problem2  Please establish a mathematical model of the discharge curve under any constant current intensity between 20A and 100A, and evaluate the precision of the model by MRE. Please give the discharge curve with the current intensity of 55A by the form and figure.

Problem3  Appendix 2 is the sampling data of the discharge curves of a battery at different decay state discharging from the full charge state under the same constant current intensity. Please predict the remaining discharge time of Decay State 3 in Appendix 2.