CUMCM-2018   Problem A

Special Clothing Design Under High Temperature Environment

When working in a high temperature environment, people need to wear special clothing to avoid thermal injuries. Special clothing usually consists of three layers of fabric material, which are denoted by Layer I, Layer II, and Layer III. Layer I is exposed to the external environment. There is a gap between Layer III and skin, which is denoted by Layer IV. The physical and geometric characteristics of each layer affect the performance of the special clothing. Appendix 1 provides some parameter values of a certain special clothing material.

For designing special clothing, a “Thermal Manikin”, whose body temperature is controlled at 37ºC, is placed in a high temperature environment in the laboratory to measure the temperature on the outside boundary of the skin of the “Thermal Manikin”. In order to reduce designing costs and shorten the designing period, you are expected to build mathematical models to determine the temperature distribution on the outside boundary of the skin. Please solve the following problems:

(1) Appendix 1 provides some parameter values of a certain special clothing material. Appendix 2 provides temperatures on the outside boundary of the skin of the "Thermal Manikin" when the environmental temperature is 75ºC, the thickness of Layer II is 6 mm, the thickness of Layer IV is 5 mm, and the working time is 90 minutes. Please establish a mathematical model to calculate the temperature distribution and generate an Excel file of temperature distribution (the file name is problem1. xlsx).

(2) Please determine the optimal thickness of Layer II to ensure that the temperature on the outside boundary of the skin of the “Thermal Manikin” does not exceed 47ºC and the time of exceeding 44ºC does not exceed 5 minutes, when the environmental temperature is 65ºC, the thickness of Layer IV is 5.5 mm, and the working time is 60 minutes.

(3) Please determine the optimal thickness of Layer II and Layer IV to ensure that the temperature on the outside boundary of the skin of the “Thermal Manikin” does not exceed 47ºC and the time of exceeding 44ºC does not exceed 5 minutes, when the environmental temperature is 80ºC and the working time is 30 minutes.

Appendix 1. Parameter values of special clothing materials

Appendix 2. Temperatures on the outside boundary of the skin of the “Thermal Manikin”