CUMCM-2019    Problem A

Pressure Control of High-pressure Fuel Pipe

The entry and ejection of the fuel oil in a high-pressure fuel pipe are the basis of many fuel engines. Figure 1 shows the working principle of a high-pressure fuel system. Fuel oil enters the high-pressure fuel pipe from inlet A, and it is ejected from nozzle B. Intermittent working process of the fuel entering and ejecting will lead to a pressure variation in the high-pressure fuel pipe. As a result, there is a deviation in the amount of the ejected fuel, and the deviation affects the efficiency of the engine.

high-pressure

fuel pump

nozzle

high-pressure fuel pipe

Figure 1   High-pressure fuel pipe

Ejection rate (mm3/ms)

Problem 1. The length of the inner cavity of a certain type of high-pressure fuel pipe is 500 mm, the inner diameter is 10 mm, and the diameter of orifice at inlet A is 1.4 mm. The fuel supply time from inlet A is controlled by a one-way valve switch. The one-way valve should be closed for 10 ms after every opening. The ejection from nozzle B works 10 times per second and the ejection time is 2.4 ms for each operation. During the ejection time, the rate of the fuel ejection from nozzle B is shown in Figure 2. The pressure provided by the high-pressure fuel pump at inlet A is 160 MPa and the initial pressure in high-pressure fuel pipe is 100 MPa. If the pressure in the high-pressure fuel pipe should keep close to 100 MPa, how to set the opening time of one-way valve? If we increase the pressure in the high-pressure fuel pipe from 100 MPa to 150 MPa within 2 s, 5 s, or 10 s respectively, then keep stable at 150 MPa, how to adjust the opening time of one-way valve?

Time (ms)

Figure 2  Injection rate

Problem 2. In reality, inlet A of the high-pressure fuel pipe is connected to the outlet of a plunger chamber of high-pressure fuel pump. The ejection from nozzle B is controlled by the needle valve of the nozzle. The schematic diagram of the actual working process of the fuel system is shown in Figure 3. Cam-driven plunger moves up and down. The relationship between the cam edge curve and the radius angle is shown in Appendix 1. The fuel in the plunger chamber is compressed as the plunger moves upward. When the pressure in the plunger chamber is greater than that in the high-pressure fuel pipe, the one-way valve at inlet A is opened and the fuel enters the high-pressure fuel pipe from the plunger chamber. The inner diameter of plunger chamber is 5 mm，When the plunger moves to the top dead center position, the residual volume is 20 mm3. When the plunger moves to the bottom stop position, the plunger chamber (including the residual volume) will be filled completely by the low-pressure fuel with pressure 0.5 MPa. The structure of the ejection nozzle is shown in Figure 4. The needle valve diameter is 2.5 mm, the seal seat is a cone with a half-angle of 9 degrees, and the diameter of the bottom nozzle is 1.4 mm. When the needle valve lift is 0, the needle valve closes. When the needle valve lift is greater than 0, the needle valve opens and the fuel is ejected from the nozzle. The relationship between needle valve lift and time in an ejection period is given in Appendix 2. Based on the given size and initial pressure of the high-pressure fuel pipe in Problem 1, as well as 10 ejections of the nozzle per second, determine the angular velocity of the cam to stable the pressure in the high-pressure fuel pipe close to 100 MPa .

high-pressure

oil pump

high-pressure oil pipe

nozzle

Figure 3 Schematic diagram of actual working process of the fuel system

Nozzle

Seal seat

Needle valve

Figure 4  Schematic diagram of the enlarged ejection nozzle

Problem 3.  Based on Problem 2, if another ejection nozzle with the same ejection rule is added, how to adjust the angular velocity of the cam to have a stable pressure in the high-pressure fuel pipe? In order to control the pressure of the high-pressure fuel pipe more effectively, it is planned to install a one-way pressure relief valve at position D (Figure 5). The outlet of the one-way pressure relief valve is a circle with a diameter of 1.4 mm. After opening the pressure relief valve, the fuel in the high-pressure fuel pipe can flow back to the external low-pressure fuel pipeline due to the pressure. This will reduce the fuel pressure in the high-pressure fuel pipe. Please give the control strategy of the high-pressure fuel pump and the relief valve.

high-pressurefuel pump

nozzle

nozzle

high-pressure fuel pipe

Figure 5  Schematic diagram of the fuel system with the valve and two nozzles

Note 1. The change of the fuel pressure is proportional to the change of the fuel density. The ratio coefficient is Eρ，where ρ is the density of the fuel. When the pressure is 100 MPa, the fuel density is 0.850 mg/mm3. E is the elastic modulus. The relationship between the elastic modulus and the pressure of the fuel is shown in Appendix 3.

Note 2. The flow rate of high-pressure fuel is Q=CA2ΔPρ，where Q is the fuel oil quantity flowing through an orifice per unit time (mm3/ms); C=0.85 is the flow coefficient; A is the area of the orifice (mm2); ΔP is the pressure difference on both sides of the orifice (MPa); ρ is the density of the fuel at the high-pressure side (mg/mm3).

Appendix 1. Cam edge curve

Appendix 2. Needle valve motion curve

Appendix 3. Relationship between elastic modulus and pressure