CUMCM-2019    Problem D

Calibration for Air Quality Data

It is well known that the air pollution is very harmful to the ecological environment and human health. By the real-time monitoring concentration of “two dusts and four gases” (PM2.5, PM10, CO, NO2, SO2, and O3), the air quality can be mastered in time and corresponding measures can be adopted to control pollution sources. Although the national monitoring station (NMS) has more accurate data for “two dusts and four gases”, it is not enough to give the real-time monitoring and forecast the air quality because of few NMS, long delay and high cost. The convenient and cheap micro-detector of air quality (see the figure below) is independently developed by a company. The micro-detector can perform grid monitoring of air quality in real-time for a certain area. It can also simultaneously monitor weather parameters such as temperature, humidity, wind speed, air pressure, and precipitation.

Since the electrochemical gas sensor used in the micro-detector may generate a certain zero drift and range drift after a long period of use, the concentration change of unconventional gaseous pollutant (gas) has cross interference to the sensor as well as the influence of weather factors on the sensor, the data of the self-built station (SBS) near the NMS collected by the micro-detector is different from that of the NMS at the same time. Therefore, it is necessary to use the hourly data of the NMS to calibrate the data of the SBS at the neighbor of the NMS.

Appendix1.csv provides hourly data of “two dusts and four gases” for a period of time of an NMS. Appendix2.csv provides data of each variable of the SBS at the neighbor of this NMS. The time interval of the SBS data is within 5 minutes. The meaning and unit of each variable are given in Appendix3.docx. Please build mathematical models to solve the following problems:

1. Make exploratory data analysis for provided data of the NMS and the SBS.

2. Analyze the factors that cause differences between the data of the NMS and the corresponding data of the SBS.

3. Use the data of the NMS to calibrate the data of the SBS by building suitable mathematical model.