CUMCM-2020    Problem A

The Furnace Temperature Curve

In the production of integrated circuit board and other electronic products, the printed circuit board with all kinds of electronic components is needed to place in the reflow furnace, and automatically weld the electronic components to the circuit board by heating. In this production process, it plays an important role for the quality of products to maintain the required temperature of each part in the reflow furnace. At present, much of the work is based on the experimental tests to control and adjust the temperatures in reflow furnace. The purpose of this topic is to study and analyze the underlying mechanism by modelling and computation to control and adjust the temperatures in reflow furnace.

Several small temperature zones are set up inside the reflow furnace, which can be divided into four large temperature zones: preheating zone, constant temperature zone, reflow zone, and cooling zone (see Fig. 1). Both sides of the circuit board are put on the conveyor belt to enter the furnace at a uniform speed for heating and welding.

Fig. 1  Sectional drawing of reflow furnace

One typical reflow furnace consists of 11 small temperature zones, the front zone, and the back zone of furnace (see Fig. 1). The length of each small temperature zone is 30.5 cm. There is a gap of 5 cm between the adjacent small temperature zones. Each length of the front and back zones is 25 cm.

Once the reflow furnace is started, the air temperature in the furnace will reach the steady state in a short time. The welding can be carried out only in the steady state. There is no special temperature control in the front zone, the back zone and the gap between small temperature zones. The temperatures in these zones and near the boundary of each temperature zone may be affected by the temperatures of the adjacent temperature zones. In addition, the temperature in the workshop is kept at 25ºC.

After setting the temperature of each temperature zone and the passing speed of the conveyor belt, the temperature at the center of the welding area in some positions can be measured by a temperature sensor. The temperature measured by the temperature sensor is called the furnace temperature curve (i.e., the temperature curve at the center of welding area). The data of furnace temperature curve in an experiment is attached, and the temperatures set in the temperature zones are 175ºC (zones 1~5)，195ºC (zone 6), 235ºC (zone 7), 255ºC (zones 8~9), and 25ºC (zones 10~11), the passing speed of conveyor belt is 70 cm/min, and the thickness of welding area is 0.15 mm. The temperature sensor starts to record when the temperature at the center of the welding area reaches 30ºC, and timing starts when the circuit board enters the reflow furnace.

In actual production, the quality of the products can be controlled by adjusting the temperature setting of each temperature zone and the passing speed of the conveyor belt. Based on the above experimental temperature settings, the temperature of each small temperature zone can be adjusted within ±10ºC. During adjustment, the temperatures in zones 1~5 are required to be consistent, the temperatures in  zones 8~9 are kept the same, and the temperatures in zones 10~11 are kept at 25ºC. The speed adjustment range of the conveyor belt is 65~100 cm/min.

In the welding production of the circuit board in the reflow furnace, the furnace temperature curve should meet some requirements, which is called process limits (see Table 1).

Table 1   Process limits

| Name | Minimum | Maximum | Unit |
| --- | --- | --- | --- |
| Slope of temperature rising | 0 | 3 | ºC/s |
| Slope of temperature drop | -3 | 0 | ºC/s |
| Time for temperature rising from 150ºC to 190ºC | 60 | 120 | s |
| Time of temperature greater than 217ºC | 40 | 90 | s |
| Peak temperature | 240 | 250 | ºC |

Your team is asked to answer the following problems:

Problem 1  Please establish a mathematical model to describe the temperature change over the welding area. Assume that the passing speed of conveyor belt is 78 cm/min, and the temperatures of the zones are 173ºC (zones 1~5), 198ºC (zone 6), 230ºC (zone 7) and 257ºC (zones 8~9), respectively. Please calculate the temperature change at the welding area center, list the temperature of the welding area center at the middle point of the zones 3, 6, 7, and the end of the zone 8. Draw the corresponding furnace temperature curve, and save the temperature of the welding area center every 0.5 s in the provided file “result1.csv”.

Problem 2  Suppose that the setting temperatures of the temperature zones are 182ºC (zones 1~5), 203ºC (zone 6)，237ºC (zone 7), and 254ºC (zones 8~9), respectively. Please give the maximum passing speed of the conveyor belt.

Problem 3  In the welding process, the temperature at the center of the welding area should not exceed 217ºC too long, and the peak temperature should not be too high. The ideal furnace temperature curve should minimize the area over 217ºC to the peak temperature (shaded part in Fig. 2). Please determine the optimal furnace temperature curve for this requirement, as well as the setting temperature in each temperature zone, the passing speed of the conveyor belt, and the corresponding area.

Problem 4  In the welding process, in addition to meeting the process limits, it is also expected that the furnace temperature curve is symmetrical as possible with the peak temperature as the center line (see Fig.2). Combined with problem 3, please further give the optimal furnace temperature curve, the setting temperature in each temperature zone，the passing speed of the conveyor belt, and the corresponding index values.

Fig. 2  Schematic drawing of furnace temperature curve