CUMCM-2020   Problem D

Automatic marking of contact profilers

Profiler is a two-coordinate measuring instrument (see Figure 1). It is composed of several components, including a working platform, a fixture, a workpiece under test, a probe, a sensor and a servo drive (see Figure 2).

Figure 1.   A type of contact profiler         Figure 2.  Schematic diagram of a contact profiler

The working principle of the contact profiler is that when the probe touches the surface of the workpiece and it slides with a constant speed, the sensor senses the geometric changes of the surface of the workpiece in the X and Z directions separately, and then it converts them into electrical signals. After amplification and other processing, the electrical signal is converted into a digital signal and it is stored in a data file (see Figure 3).

- Schematic diagram of the contact                     (b) Digital signal

profiler measurement

Figure 3.  Working principle of the contact profiler

Ideally, the contour curve should be smooth. However, due to the facts such as probe contamination, probe defect and inaccurate scanning position of the contact profiler, the detected contour curve usually appears rough and uneven (see the local enlarged figure in Figure 3(b)), which affects the accurate marking of the workpiece shape.

To simplify the above issues, we assume that the contour of the workpiece is a plane curve composed of straight lines and arcs (see Figure 4). Please establish mathematical models to solve the following problems based on the profiler measurement data provided in Annex 1 (the horizontal as well as the tilt measurement data of workpiece 1) , Annex 2 to Annex 4 (multiple measurement data of workpiece 2) .

Problem 1. The “level” sheet in Annex1 is the measurement data of workpiece 1 under the horizontal measurement. Its contour line is shown in Figure 4. Please determine the parameter values of the contour line: the width of the notches (such as x1, x3, etc.), the radius of the arcs ( such as R1, R2, etc.), the distance between the circles (such as c1, c2, etc.), the length of the arcs, the length of the horizontal (such as x2, x4, etc.), the length of the oblique lines, the angle between the oblique lines and the horizontal lines (such as∠1, ∠2, etc.), the height of the herringbone line (z1).

Figure 4.  The contour line of workpiece 1 measured in a horizontal state

Problem 2. For different measurements of a same workpiece, the parameters of the contour line will be different due to the different angles and positions of the workpiece. The “down” sheet in Annex 1 gives the measured data of the contour line of workpiece 1 in the state of a special angle and some horizontal displacements. Please calculate the angle of the workpiece during the measurement and make the corresponding horizontal correction. After the data correction, complete the task in Problem 1 and compare the differences between the calculated values of each parameter of workpiece 1 in the two measured states.

Problem 3. When the workpiece is tested for many times, there exist deviations of the angle at which the workpiece is placed and the starting as well as the ending points of the measurement each time, which leads to the fact that each measurement is actually a part of the whole workpiece. Annex 2 provides ten groups of measurement data of workpiece 2. Please complete the following tasks based on these data: (1) confirm the tilt angle of workpiece 2 during each measurement; (2) Mark the parameter values of the contour line of workpiece 2 (the same as in Problem 1); (3) Draw the complete outline of workpiece 2.

Problem 4. In order to mark the parameter values of workpiece 2 more accurately, Annex 3 and Annex 4 provide nine groups of local measurement data of the circles and the angles of workpiece 2, respectively. Please use these data to correct the conclusions of Problem 3 and further modify the complete contour line of the workpiece.