CUMCM-2020    Problem E

Intelligent Management of the Campus Water Supply System

Campus water supply system is an important part of campus utilities. In order to ensure the normal operation of the campus water supply system, the school needs to invest a lot of human, material and financial resources. With the development of science and technology, smart water meters have been widely used in campus, so that a large amount of real-time water supply system operation data can be obtained. The Service Department wishes to promptly find and solve some problems in the water supply system based on these data to improve the campus service and management by using mathematical models and data mining technologies.

The attachments are the water meter hierarchy table of the campus and the records of all water meters (at intervals of time, such as 15 minutes) and the corresponding water data in four quarters of 2019. Please use these information and data to build some mathematical models and discuss the following questions:

- Try to analyze the data of each water meter, and give the water using characteristics of the different functional areas (such as dormitories, teaching buildings, office buildings, canteens, etc.).

- Based on the water meter hierarchy table of the campus, please try to establish the relationship model of water meters’ records, and use the giving data to discuss the error of the model.

- The leakage of the pipeline network is a serious problem in water supply system. According to the documents, the average water loss is about 5% in a well-maintained public water supply network, while in older pipe networks, there is more water loss. Try to build a mathematical model based on data to analyze the leakage of the campus water supply network.

- Underground water pipe leakage is not easy to be found, it takes a lot of manpower to detect and locate the leakage of the water supply pipe. If the location of leakage can be detected from the real-time data of the water meters, it will be extremely beneficial. Please help the school to solve this problem.

- Pipe network maintenance requires a certain amount of labor and material costs, but at the same time the leakage of the pipe network can be reduced. Please give a pipe network leakage repair decision plan according to the above results and the cost of water and maintenance, such that the campus annual cost for the water supply system is minimal.